The Missing Trace

A Real-Time Data Integration and Process Optimization CTF Challenge

CITM-501: Information Systems Infrastructure

Assignment 6.6 – Capture the Flag Activity

Challenge Author: Meghana Sinha – 2953222

Team size: 3–4 players  |  Target completion time: 45 minutes  |  Version 2

Challenge Objective

NorthBridge Health Supply is a fictional distributor of serialized medical products. Every pallet received at the warehouse is scanned, posted to the ERP system, and passed in real time through an integration layer to a cloud traceability platform. Regulators and hospital customers rely on that platform to prove where each serialized item has been.

At 09:30 the following ticket arrives at the service desk:

INC-20931  Priority: High  Opened: 09:30  Caller: Warehouse Operations Lead

"The traceability dashboard is showing 8 receipts missing since about 09:10. They are all in

the ERP. My team has been resending and rescanning to try to push them through. Please fix

this before the 11:00 hospital shipment cut-off."

Your team is the integration support group. Using only the evidence in this packet, determine which component failed, exactly when it started failing, how many unique business transactions actually need to be recovered, and which long-term design prevents a repeat.

How to Play as a Team

The evidence is split into four work streams. Assign one role per person (combine roles 3 and 4 for a team of three). Each role contributes one part of the flag, so no one can finish alone. Plan to work in parallel for the first 20 minutes, then regroup and compare findings.

| Role | Primary Evidence | Delivers to the Team |
| --- | --- | --- |
| 1. Data Reconciliation Analyst | A (Transactions), G (Scanner & Ops Log) | Which missing receipts are unique, which are duplicates, and which never reached the integration layer |
| 2. Log Analyst | B (Hub Log), C (Proxy Log), H (Change Calendar) | The exact time the root-cause component began failing, and what changed |
| 3. Infrastructure Analyst | D (Connectivity Tests), E (Vendor Status) | Which network path and component is broken, and which are healthy |
| 4. Solutions Architect | F (Architecture Options) + findings from roles 1–3 | The option that prevents recurrence without creating new data-quality risk |

Rules

Use only the evidence included in this packet. No outside lookups are needed.

The component that reports an error is not necessarily the component that caused it. Compare evidence across layers.

A “missing” receipt on the dashboard is not automatically a failed integration message. Trace each one.

Separate retries, manual resends, and rescans from unique business transactions.

Your final answer is a single flag in the exact format shown at the end. It is checked character-for-character.

Two optional hints are available at the end of the packet. Note the time if you use them.

Current System Flow

| 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
| Warehouse Scanner | ERP Receiving | Integration Hub | Legacy App Proxy | Traceability Cloud (TraceCloud) | Operations Dashboard |
| Wi-Fi VLAN 30 → HTTPS | Posts receipt, creates a Match ID | Holds and retries messages (3×, 30 s) | Forwards hub → cloud, port 8443 | Vendor API, port 443 | Reads TraceCloud data |

ERP creates a Match ID from the purchase order and the serial numbers on the pallet. If two rows have the same Match ID, they are the same physical pallet — even if the transaction IDs are different.

Business Impact

Operations uses the traceability dashboard to confirm that serialized products were received and are available for downstream picking and shipping. Products that are not visible cannot ship. When receipts appear missing, staff tend to resend or rescan, which creates duplicate serial records in the cloud platform if those duplicates are later replayed. Duplicate serial records must be manually investigated and can trigger compliance holds.

Evidence A – Transaction Timeline (ERP + Dashboard export, 09:29)

| Txn ID | ERP Time | ERP Status | PO | Match ID | Hub Status | Cloud Ack | Dashboard |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1001 | 09:02:14 | Posted | PO-5510 | RX-88410 | Forwarded | ACK-200 | Visible |
| T1002 | 09:05:29 | Posted | PO-5511 | RX-88411 | Forwarded | ACK-200 | Visible |
| T1003 | 09:06:11 | Posted | PO-5512 | RX-88412 | Parked | None | Missing |
| T1004 | 09:08:08 | Posted | PO-5513 | RX-88415 | Parked | None | Missing |
| T1005 | 09:10:03 | Rejected (VAL-112) | PO-5514 | — | Not received | None | Missing |
| T1006 | 09:11:30 | Posted | PO-5515 | RX-88419 | Parked | None | Missing |
| T1007 | 09:14:17 | Posted | PO-5516 | RX-88421 | Parked | None | Missing |
| T1008 | 09:18:04 | Posted | PO-5515 | RX-88419 | Parked | None | Missing |
| T1009 | 09:21:39 | Posted | PO-5517 | RX-88426 | Parked | None | Missing |
| T1010 | 09:23:01 | Posted | PO-5512 | RX-88412 | Parked | None | Missing |
| TEST-DIRECT | 09:25:10 | N/A | — | — | Health test | HTTP 200 | N/A |

VAL-112 = the ERP rejected the receipt because the lot number was blank. Rejected receipts are never sent to the hub. Parked means the hub tried three times, failed, and is holding the message.

Evidence B – Integration Hub Log

08:00:00  CONFIG  retry_max=3  retry_interval=30s  dedupe_check=OFF  parked_queue_alert=OFF

09:02:15  MSG T1001  key=RX-88410  route=legacy-proxy:8443  attempt=1  result=HTTP_200  ack=ACK-200

09:05:30  MSG T1002  key=RX-88411  route=legacy-proxy:8443  attempt=1  result=HTTP_200  ack=ACK-200

09:06:12  MSG T1003  key=RX-88412  route=legacy-proxy:8443  attempt=1  result=HTTP_504_GATEWAY_TIMEOUT

09:06:42  MSG T1003  key=RX-88412  route=legacy-proxy:8443  attempt=2  result=HTTP_504_GATEWAY_TIMEOUT

09:07:12  MSG T1003  key=RX-88412  route=legacy-proxy:8443  attempt=3  result=RETRY_EXHAUSTED  status=PARKED

09:08:09  MSG T1004  key=RX-88415  route=legacy-proxy:8443  attempts=3  result=RETRY_EXHAUSTED  status=PARKED

09:11:31  MSG T1006  key=RX-88419  route=legacy-proxy:8443  attempts=3  result=RETRY_EXHAUSTED  status=PARKED

09:14:18  MSG T1007  key=RX-88421  route=legacy-proxy:8443  attempts=3  result=RETRY_EXHAUSTED  status=PARKED

09:18:05  MSG T1008  key=RX-88419  source=MANUAL_RESEND  attempts=3  result=RETRY_EXHAUSTED  status=PARKED

09:21:40  MSG T1009  key=RX-88426  route=legacy-proxy:8443  attempts=3  result=RETRY_EXHAUSTED  status=PARKED

09:23:02  MSG T1010  key=RX-88412  source=ERP_RECEIPT  attempts=3  result=RETRY_EXHAUSTED  status=PARKED

09:25:10  TEST-DIRECT  route=tracecloud-api:443 (temporary rule FW-TMP-77)  result=HTTP_200  health=OK

Evidence C – Legacy App Proxy Log

08:52:10  CHG-4471  OS security patch applied; relay configuration reloaded

08:52:11  truststore=/opt/lap/certs rebuilt from baseline image  entries=112

08:55:47  graceful restart completed; existing upstream pool connections retained  max_age=600s

09:02:15  relay T1001  upstream=tracecloud-api:443  pooled_conn=7  status=200

09:05:30  relay T1002  upstream=tracecloud-api:443  pooled_conn=3  status=200

09:05:47  pool=tracecloud  all pooled connections reached max_age; opening new upstream connections

09:05:47  upstream TLS handshake failed: unable to get local issuer certificate (TraceCloud Issuing CA G2)

09:05:47  connector_pool=tracecloud  status=UNAVAILABLE

09:06:12  inbound session accepted from integration-hub

09:06:42  upstream connection not established before timeout (30s); returned 504 to client

09:08:09  upstream connection not established before timeout (30s); returned 504 to client

09:11:31  upstream connection not established before timeout (30s); returned 504 to client

...    (same pattern repeats for every relay request through 09:23:02)

Evidence D – Connectivity Tests (Network Team, 09:25–09:28)

| Source | Destination | Test | Result |
| --- | --- | --- | --- |
| Warehouse scanner (VLAN 30) | ERP Receiving :443 | HTTPS | Success |
| ERP server | Integration Hub :443 | TCP | Success |
| Integration Hub | Legacy App Proxy :8443 | TCP | Success (4 ms) |
| Integration Hub | Legacy App Proxy :8443 | POST /relay/tracecloud | HTTP 504 after 30 s |
| Legacy App Proxy | TraceCloud API :443 | TCP | Success (22 ms) |
| Legacy App Proxy | TraceCloud API :443 | TLS handshake | Failed – certificate verify failed (unknown issuer) |
| Integration Hub | TraceCloud API :443 | HTTPS health check via FW-TMP-77 | HTTP 200 – certificate chain valid (Issuing CA G2) |
| User workstation | Operations Dashboard | HTTPS | Success |

Evidence E – External Platform Status (TraceCloud Status Page, 09:27)

All systems operational. No active incident. API health endpoint responding normally. No maintenance window in progress. Inbound processing normal for all regions during the last 24 hours.

Notice (posted Sep 28): TraceCloud replaced its security certificate with a newer one called “TraceCloud Issuing CA G2.” Any customer that keeps its own list of trusted certificates must add the new G2 certificate. Everyday browsers and public lists already include it.

Evidence F – Architecture Options

Option A – Keep sending data through the old proxy. Give the hub more time (30 seconds to 2 minutes) and more retries (3 to 10) so short delays can clear on their own.

Option B – Connect the ERP straight to the cloud and shut down both the hub and the old proxy, so there are fewer systems that can break.

Option C – Keep the hub so IT can still hold and retry messages. Stop using the old proxy and send data from the hub straight to the cloud on an approved secure connection. Ignore repeat sendings of the same pallet so resends do not create duplicate records, and alert the team when messages sit unsent for too long.

Option D – Keep the hub and stop using the old proxy; send data from the hub straight to the cloud. Add a job that automatically resends every stuck message every 15 minutes.

Evidence G – Warehouse Scanner & Operations Log

09:06:05  WH-SCN-02  user=wh.recv1  PO-5512  pallet scanned  serials=48  lot=L24-0912

09:10:01  WH-SCN-04  user=wh.recv3  PO-5514  pallet scanned  serials=36  lot=<blank>  label damaged

09:11:24  WH-SCN-02  user=wh.recv1  PO-5515  pallet scanned  serials=60  lot=L24-0915

09:17:40  OPS-DESK   user=wh.ops2   "PO-5515 not on dashboard - used ERP Resend button"

09:20:55  WH-SCN-04  user=wh.recv3  PO-5512  pallet RESCANNED  serials=48  lot=L24-0912

note="not visible on dashboard, scanning again"

09:24:30  OPS-DESK   user=wh.ops2   "Told team to stop rescanning until IT responds"

Evidence H – Change Calendar (Today)

| Change | Window | System | Description | Status |
| --- | --- | --- | --- | --- |
| CHG-4460 | 08:30–08:40 | Firewall | Rule update on warehouse wireless VLAN 30 (scanner network) | Completed |
| CHG-4471 | 08:50–09:00 | Legacy App Proxy | Monthly OS security patch and config reload. Risk: Low. “No user impact expected.” | Completed |
| CHG-4475 | 22:00–23:00 | ERP | Monthly database index maintenance | Scheduled |
| INC-20410 | Sep 28 | Legacy App Proxy | Manually imported TraceCloud Issuing CA G2 into proxy truststore after vendor rotation | Closed |

Your Tasks

Root cause. Identify the component that is the root cause of the failure (use the component codes below).

Onset. Determine the time (HH:MM, 24-hour) at which the root-cause component itself first became unable to deliver data.

Recovery scope. Determine how many unique business transactions must be replayed through the integration layer. Do not count resends, rescans, or receipts that never reached the hub.

Immediate recovery. Describe the safest immediate recovery action, including what to do with the duplicate and non-integration items.

Long-term design. Choose the architecture option (A, B, C, or D) that best prevents recurrence while preserving real-time processing and data integrity.

Control. State one monitoring or control improvement that would have detected this problem before operations did.

Component Codes

| Component | Code |
| --- | --- |
| Warehouse Scanner | SCANNER |
| Firewall | FIREWALL |
| ERP Receiving | ERP |
| Integration Hub | HUB |
| Legacy App Proxy | LEGACYPROXY |
| Traceability Cloud | TRACECLOUD |
| Operations Dashboard | DASHBOARD |

Final Flag Format

FLAG{<COMPONENT_CODE>_<ONSET_HHMM>_<UNIQUE_REPLAY_COUNT>_<OPTION_LETTER>}

Formatting rules: uppercase, no spaces, no colon in the time. Example of the format only (not the answer): FLAG{ERP_1342_3_A}.

Team Answer Sheet

Submit the flag for verification. Complete the reasoning below so the playtest reviewer can confirm the flag came from analysis rather than guessing.

Team members and roles:

__________________________________________________________________________________________________

1. Root-cause component and the two strongest pieces of evidence:

__________________________________________________________________________________________________

__________________________________________________________________________________________________

2. Onset time and the log line that proves it:

__________________________________________________________________________________________________

3. Transactions to replay (list the IDs) and transactions excluded (with reason):

__________________________________________________________________________________________________

__________________________________________________________________________________________________

4. Immediate recovery action:

__________________________________________________________________________________________________

__________________________________________________________________________________________________

5. Architecture option and why the other three are weaker:

__________________________________________________________________________________________________

__________________________________________________________________________________________________

6. Monitoring / control improvement:

__________________________________________________________________________________________________

FINAL FLAG:

__________________________________________________________________________________________________

Start time / finish time / hints used:

__________________________________________________________________________________________________

Optional Hints

Hint 1 (after 20 minutes): Line up the Match ID column. Two different transaction IDs can describe the same physical pallet. Also check whether every missing receipt actually reached the hub.

Hint 2 (after 30 minutes): The hub records when it noticed the failure, not when the failure began. Find the earliest line where the failing component reports its own problem, and ask why it happened then rather than right after the restart.

References

CTF101. (n.d.). CTF 101. https://ctf101.org/

CTFd. (n.d.). What is Capture the Flag? https://ctfd.io/whats-a-ctf/